|  | SPECIFICATION |  |
| --- | --- | --- |

Free Declaration

Please read through the manual carefully before using the product and operate it according to the manual. It is advised that you should keep this manual for future reference.

Do not disassemble the device or remove the seal label from the device, doing so will void the product warranty provided by Shenzhen alacrity barcode technology Co.,Ltd .

All pictures in this manual are for reference only and actual product may differ. Regarding to the product modification and update, Shenzhen alacrity barcode technology Co.,Ltd . reserves the right to make changes to any software or hardware to improve reliability, function, or design at any time without notice. The information contained herein is subject to change without prior notice.

The products depicted in this manual may include software copyrighted by Shenzhen alacrity barcode technology Co.,Ltd  or third party. The user, corporation or individual, shall not duplicate, in whole or in part, distribute, modify, decompile, disassemble, decode, reverse engineer, rent, transfer or sublicense such software without prior written consent from the copyright holders.

This manual is copyrighted. No part of this publication may be reproduced, distributed or used in any form without written permission from Vancode.

.

Shenzhen alacrity barcode technology Co.,Ltd . reserves the right to make final interpretation of the statement above.

Shenzhen alacrity barcode technology Co.,Ltd .3F, Building A3, Huazhan Science and Technology Industrial Park, Qianqiuling Village, Qingxi Town, Dongguan City, Guangdong Province, China   http://www.Vancode.cn

Revision History

| Version | Description | Date |
| --- | --- | --- |
| V1.0 | Initial release. | July 21, 2016 |

Contents

Chapter 1 Getting Started	7

Introduction	7

Feature	8

Application Field	8

Manual Introduction	9

Optics	10

Module Size	13

Data Interface	14

Chapter 2 Barcode Scanner Parameter	15

Physical Parameter	15

Performance Parameter	16

Depth of field	17

User Environment	17

Chapter 3 System Setting	18

Introduction	18

Scanning Introduction	18

Factory Defaults	19

Check the version	19

Scan mode	20

Cell Mode	21

Inverse Color	22

Illumination LED	23

Buzzer Setting	24

Barcode Scanning Delay	26

Chapter 4 Communication Settings	27

Introduction	27

USB Communication Mode	27

Convert Case	28

USB COM Port Emulation	28

RS232 Interface	29

Chapter 5 Data Edition	32

Introduction	32

Terminating Character Setting	33

Custome Prefix and Suffix	34

Chapter 6 Symbologies	36

Introduction	36

General Setting	36

EAN-8	37

EAN-13	39

Codabar	41

Code 39	42

Code 93	43

Code 128	44

GS1-128	44

UPC-A	45

UPC-E	47

Interleaved 2 of 5	49

Industrial 2 of 5	50

Matrix 2 of 5	50

MSI	51

Telepen	51

China Postal Code	52

QR Code	53

Data Matrix	53

PDF 417	54

Micro PDF 417	54

Maxi code	55

Aztec	55

Hanxin	56

Chapter7 Serial Commands	57

Introduction	57

Command Format	57

Trigger Command	57

An example of sending command	58

Chapter 8 Appendix 	59

Appendix（Byte Code）	59

Appendix（ASCII Table）	60

Appendix（Regular Instruction Commands）	65

Chapter 1 Getting Started

Introduction

MJ-3000 is a sort of 2D barcode engine with great performance. This model uses the domestic leading imaging technology with the super decoding capacity and data acquisition ability, can easily scan the high-density 1D codes, 2D codes and screen codes.

Newest compact design integrates scanner and Chroma module. The scanner can quickly acquire image by using advanced pulsed light and high-speed shutter sensor. With strong micro processor, it only takes a few millisecond to decode 1D and 2D codes. It will also improve the work efficiency of related industry with this excellent scanning capability.

Features

- Area-image scanning: Domestic leading 2d Area-image scanner, it can easily to scan the printing codes and screen codes.

- Integrated:Compact size fits for many situations. Easy to operate, support TTL, USB interfaces.

- Wide Vision high lumen: with wide vision angel and wide decode range.Accurate aimer and high lumen Led help it easy to scan the codes in dark environment.

- Low Consumption:Low consumption to support the wireless device.

- Multiple systems support: Support Linux, MAC, Windows, Android systems perfectly.

Application Field

Barcode Scanner, Wireless Collector, Self-service vending machine, Self service terminal, Queuing equipment, Lottery machine etc.

Manual Introduction

MJ-3000 General Requirements

ESD Protection: ESD protection has been taken into account when designing the MJ-3000 and the engine is shipped in ESD safe packaging. Always exercise care when handing the engine outside its package.Be sure grounding wrist straps and properly grounded work areas are used.

Dust and Dirt:The MJ-3000 must be sufficiently enclosed to prevent dust particles from gathering on the image and lens.Dust and other external contaminants will eventually degrade the engine’s performance .

Ambient Environment: MJ-3000 Working temperature :-20℃~50℃，Storage temperature:-40℃~70℃，Humidity:5%-95%（non-condensing ).

Thermal Considerations :Electronic components in the VS3122 will generate heat during the course of their operation. Operating the EM3080 in continuous mode for an extended period may cause temperature to rise on CIS and decoder chips. Overheating can degrade image quality and affect scanning performance.Given that,the following  precautions should be taken into consideration when integrating the VS3122:

1.Reserve the sufficient space for good air circulation in the design.

2.Avoid Wrapping the VS3122 with thermal insulation materials such as rubber.

Installation Orientation:The following figure illustrates a front view of the MJ-3000 after installation, with the engine’s decoder board on the top,and the illumination and lens on the front.

Optics

Window Placement

The window should be positioned properly to let the illumination and the aiming beams pass through as much as possible and no reflections back into the engine(reflections can degrade the reading performance).

Window should be mounted close to the front of the engine(parallel).The maximun distance is measured from the front of the engine housing to the farthest surface of the window.In order to reach the better reading performance,the distance from the front of the engine housing to the furthest surface of the window should not exceed 1.5mm and the distance from the front of the engine housing to the nearest surface of the window should not exceed 0.5mm.

If the window is required to be in a tilted position,the above distance requirements should be met and tilt angle should ensure no reflections back into the lens.

Window Material and Color

Illumination wavelengths and CIS’s responsiveness (mainly to wavelengths of red light) should be taken into consideration when choosing window material and color, in order to achieve the possible highest spectral transmission, lowest haze level and homogeneous refractive index. It is suggested to use PMMA or optical glass with spectral transmittance over 95% and haze less than 1%. Whenever to use an anti-reflection coating or not depends on the material and application needs.

Window Size

The window must not block the field of view and should be sized to accommodate the aiming and illumination envelopes shown below.

Structure Size

The illumination below show the mechanical mounting dimensions for the MJ-3000.The structural design should be leave some space between components.

Front View（unit：mm）

Left View（unit：mm）

Bottom View（unit：mm）

Data Interface

Interface Definition

The table below listed the names and signal definition of 12PIN of MJ-3000.

|  | PIN# | Signal | I/O | Definition |
| --- | --- | --- | --- | --- |
|  | 1 | VCC | - | Current Source |
|  | 2 | VCC | - | Current Source |
|  | 3 | GND | - | Ground |
|  | 4 | RX | Input | TTL-RS232 Input |
|  | 5 | TX | Output | TTL-RS232 Output |
|  | 6 | D- | Input/Output | USB D- Signal |
|  | 7 | D+ | Input/Output | USB D+ Signal |
|  | 8 | PWR_DWN | Output | Ground |
|  | 9 | BUZ | Output | Buzzer Output |
|  | 10 | LED | Output | LED decoding output signal |
|  | 11 | Wake | Input | To wake up signal,Keep 100-500μs low level signal to trigger the engine. |
|  | 12 | TRIG | Input | To trigger the input signal, keep 10ms signal to trigger code reading. |

Chapter 2  Scanner Parameter

Physical Parameter

| Physical Parameter |  |  |
| --- | --- | --- |
|  | Device Weight | <5g |
|  | Device Dimension | 21.2mm L * 12.0mm H * 11.2mm D（Scanner） |
|  | Interface | FFC 12pin 0.5pitch |
|  | Interface | USB/TTL |

Performance Parameter

| Performance Parameter |  |  |
| --- | --- | --- |
|  | Light Source | 617nm LED（aimer）、6500K LEDs（illumination） |
|  | Sensor | 640*480 CMOS |
|  | Processor | ARM32-bit |
|  | Resolution | ≥5mil/0.127mm@PCS90% |
|  | Print Contrast | 25%PCS |
|  | Scan Angles | Test Conditions：CODE39,10mil/0.25mm,PCS90% |
|  |  | Pitch:±65° |
|  |  | Roll: 0-360° |
|  |  | Skew: ±60° |
|  | Promoting Mode | Buzzer，Indicator(LED) |
|  | Scan Mode | Trigger Button、Command Trigger、Inductive Trigger |
|  | Power adapter | output：DC5V,1A input：AC100~240V，50~60Hz |
|  | Power supply | 3.3V DC±5%@280mA |
|  | Motion Tolerance | 63.5CM/S |
|  | Ambient illumination | 100000Lux Max |
|  | Decode Capability | 1D：EAN-8, EAN-13, Codabar, CODE 39, CODE 93, CODE 128, China Post, GS1-128, UPC-A, UPC-E, UCC/EAN 128, ISBN/ISSN, ISBT , Interleaved 2 of 5，Standard 2 of 5, Matrix 2 of 5，Industrial 2 of 5 , MSI, RSS, ITF14, Telepen, etc. 2D：QR Code, Micro QR Code, Data Matrix, PDF417, Micro PDF417, Maxicode, Aztec, Hanxin, etc. |

Depth of field

|  | Range of decoding |  |  |  |
| --- | --- | --- | --- | --- |
|  | Type of Code | Code dimension | Nearest | Farthest |
|  | Code 39 | 0.127mm(5mil) | 50mm | 90mm |
|  | Code 39 | 0.5mm(20mil) | 40mm | 200mm |
|  | UPC-A | 0.33mm(13mil) | 40mm | 180mm |
|  | QR Code | 20.0mil | 40mm | 200mm |
|  | Data Matrix | 10mil | 40mm | 110mm |
|  | PDF 417 | 6.8mil | 50mm | 90mm |

Test condition：25℃ indoor, ambient illumination 200LUX，PCS=0.9

User Environment

|  | User Environment |  |
| --- | --- | --- |
|  | Operating Temperature | -20℃ to 50℃ |
|  | Storage Temperature | -40℃ to 80℃ |
|  | Relative Humidity | 5% to 95%（non-condensing） |
|  | Drop test | 1.2 meter |
|  | Temperature test | 30 minutes for high Temp., 30 minutes for low Temp., |
|  | Highest temp | 60℃ |
|  | Lowest temp | -20℃ |
|  | Shock resistance | 10H@125RPM |

Chapter 3 System Setting

Introduction

The scanner can be configured by scanning programming barcodes.

Manual introduction

Manual scanning mode,please follow the scanning steps:

1.	Press and hold the trigger button, then the light is activated, and appear the red aiming light and the white led light.

2.	Aim the center of the codes by red aiming light, to move the scanner between the codes to find the best scanning distance.

3.	Decode successfully when you hear the buzzer sound and the red light will be off,the codes will be transmitted to the host.

Note：During scanning the same series codes, you will find there will be a highly success rate between the scanner and the codes in some distance, this refers to the best reading distance.

Factory Defaults

Scanning the following barcode can restore the scanner to the factory defaults.

You may need to reset your scanner when:

1. scanner is not properly configured so that it fails to decode barcodes;

2. you forget previous configuration and want to avoid its impact;

3. functions that are rarely used have been enabled for the time being.

Restore All Factory Defaults

Check the version

Scanning the code to check the scanner version.

Check the version

Scanning Mode Setting

Trigger Mode

If the trigger mode is enabled, you could activate the scanner by providing an external hardware trigger. When in manual trigger mode, the scanner scans until a barcode is read, or until the hardware trigger is released.

Trigger Mode(Default)*

This set the scanner to work in Presentation mode. The scanner will be triggered when the image pass by.

Presentation Mode

Cell Phone Mode

If the Cell Phone Mode is enabled, the engine activates a special capturing image and illumination session. If you want to enable Cell Phone reading in Trigger Mode, please scan below barcode:

Trigger Mode with Cell Phone reading

If you want to enable Cell Phone reading in Presentation Mode, please scan below barcode:

Presentation Mode with Cell Phone reading

Inverse color

Scan the "Inverse On " code when you need to scan some abnormal printing codes.

Off(Default)*

Inverse On

Inverse&Normal Both On

Illumination led

Illumination setting for Trigger Mode. Illumination Level 4 is highest illumination level.

| Illumination Level 4(Default)* | Illumination Level 3 |
| --- | --- |
| Illumination Level 2 | Illumination Level 1 |
| Illumination OFF |  |

Buzzer Setting

On and off

There will be a prompting sound after scanning successfully, and the buzzer can be turned off for user requirements.

| Buzzer On(Default)* | Buzzer Off |
| --- | --- |

Buzzer Tone

Low

Middle

High(Default)*

Buzzer Number

| Once(Default)* | Twice |
| --- | --- |
| Three times | Four times |
| Five times | Six times |

Barcode Scanning Delay

To set delay by scanning the programming code below, you can control the scanning delay after being triggered.

No Delay(Default)

Delay 500ms

Delay 2000ms

Chapter 4 Communication Settings

Introduction

The scanner provides a USB interface, USB-COM interface, RS232 interface to communicate with the host device. The host device can receive scanned data and send commands to control the scanner or to access/alter the configuration information of the scanner via the USB interface.

USB Communication Mode

To connect the scanner with the host by USB interface, you can scan USB HID-KBW programming code to set scanner to USB output mode, and you don’t need to install any drivers, just plug and play.

USB HID-KBW(Default)

Convert Case

This parameter is valid when the Covert Case is set. When the Convert All to Lower Case feature is enabled, barcode data “aBC123” is transmitted as “abc123”.

No Case Conversion(Default)*

| Convert All to Upper Case | Convert All to Lower Case |
| --- | --- |

USB COM Port Emulation

This feature allows the host to receive data in the way as a serial port does. However, you need to set communication parameters on the scanner to match the Host requirements. A driver is required for this feature.

USB COM Port Emulation

Note：In USB COM Port Emulation, the port protocol parameter of the scanner can match the port parameter of application of the host automatically.

RS232 Interface

Serial communication interface is usually used to connect the scanner to a host device (like PC, POS).When the scanner is connected to a host device through its RS-232 interface, you need to set communication parameters to match the host device.

RS232(Default)*

Default serial communication parameters are listed below. Make sure all parameters match the host requirements.

| Parameter | Default |
| --- | --- |
| Serial Communication | Standard TTL-RS232 |
| Baud Rate | 115200 |
| Parity Type | None |
| Data Bits | 8 |
| Stop Bits | 1 |

Baud Rate

Baud rate is the number of bits of data transmitted per second. Set the baud rate to match the Host requirements.

| 2400 | 4800 |
| --- | --- |
| 9600 | 19200 |
| 38400 | 57600 |
| 115200(Default)* |  |

Data Bits&Parity Check&Stop Bits

None Parity /8 Data Bits/1 Stop Bit(default)

Even Parity /8 Data Bits/1 Stop Bit

Odd Parity /8 Data Bits/1 Stop Bit

Chapter 5 Data Edition

Introduction

After a successful barcode read, a string containing numbers, letters or symbols will be returned. In real applications, barcode data may be found insufficient for your needs. You may wish to include additional information such as barcode type, data acquisition time or delimiter in data being scanned. Adding extra information to printed barcodes does not seem like a sensible solution since that will increase the barcode size and make them inflexible. Instead, we come up with the idea of appending prefix and suffix to the data without making any change to barcodes. We will show you how to conduct the configuration in the following sections.

Note: Customized data: <Prefix> <Data><Suffix>

Terminating Character Setting

Corresponding terminating character can be added during using the scanner to meet the user's requirements.

Add Return CR

.

Add Newline LF

Add Return+Newline

Add TAB（ＨＴ）

Custom Prefix and Suffix

Users can custom the prefix and suffix of the output code for your requirements. For example, when you add prefix “VC” to barcode “123”, the host will receive “VC123”. When you add suffix “DE” to barcode “123”, the host will receive “123DE”.

Set Custom Prefix

To set a custom prefix, scan the numeric barcodes which representing the hexadecimal values of a desired prefix in Appendix B, then scan“Set Custom Prefix”, third scan the corresponding Byte Bit which representing the hexadecimal values of a desired prefix in Appendix A, finally scan “Save”. Refer to Appendix B:ASCII table for hexadecimal values of characters.

Example：Set the custom prefix  to “OK”.

A、Check the hex value of “OK”in the ASCII table.(“OK”:4F 4B)

B、Scan“Set Custom Prefix”programming code

C、Scan the numeric barcode “9”,”9”,”4”，”F”，”4”，”B”in Appendix A

D、Scan“Save”.

Set Custom Prefix

| Save | Not Save |
| --- | --- |

Set Custom Suffix

To set a custom suffix, scan the numeric barcodes which representing the hexadecimal values of a desired suffix in Appendix B, then scan“Set Custom Suffix”, third scan the corresponding Byte Bit which representing the hexadecimal values of a desired suffix in Appendix A, finally scan “Save”. Refer to Appendix B:ASCII table for hexadecimal values of characters.

Example：Set the custom Suffix  to “VC”。

A、Check the hex value of “VC”in the ASCII table.(“VC”:56 43)

B、Scan“Set Custom Suffix”programming code

C、Scan the numeric barcode “9”,”9”,”5”，”6”，”4”，”3”in Appendix A

D、Scan“Save”

Set Custom Suffix

| Save | Not Save |
| --- | --- |

Save and Clear

Scan “Clear all Prefix and Suffix”programming code, you can clear all prefix and suffix you have set, but not include Terminating Character.

Clear all Prefix and Suffix*

Chapter 6 Symbologies

Introduction

Every symbology (barcode type) has its own unique attributes. This chapter provides programming barcodes for configuring the scanner so that it can identify various barcode symbologies. It is recommended to disable those that are rarely used to increase the efficiency of the scanner.

General Setting

If “Disable All Symbologies”feature is enabled, the engine will not be able to read any non-programming codes except the programming codes.

Enable All Symbologies

Disable All Symbologies

EAN-8

Restore Factory Defaults

Restore the Factory Defaults of EAN-8(Default)

Enable/Disable EAN 8

| Enable EAN-8(Default)* | Disable EAN-8 |
| --- | --- |

Transmit Check Digit

EAN-8 is 8 digits in length with the last one as its check digit used to verify the integrity of the data. The default mode opens “transmit EAN-8 Check Digit”. Users can scan the code below to choose it.

| Transmit EAN-8 Check Digit(Default)* | Do Not Transmit EAN-8 Check Digit |
| --- | --- |

Add-On Code

An EAN-8 barcode can be augmented with a two-digit or five-digit add-on code to form a new one. In the examples below, the part surrounded by blue dotted line is an EAN-8 barcode while the part circled by red dotted line is add-on code.

| Enable 2-Digit Add-On Code | Disable 2-Digit Add-On Code(Default)* |
| --- | --- |
| Enable 5-Digit Add-On Code | Disable 5-Digit Add-On Code(Default)* |

EAN8 Addenda Separator

When this feature is enabled, there is a space between the barcode and addenda. When this feature is disabled, there is no space.

| Enable EAN8 Addenda Separator(Default)* | Disable EAN8 Addenda Separator |
| --- | --- |

EAN-13

Restore Factory Defaults

Restore the Factory Defaults of EAN-13

Enable/Disable EAN 13

| Enable EAN-13(Default)* | Disable EAN-13 |
| --- | --- |

Transmit Check Digit

EAN-13 is 13 digits in length with the last one as its check digit used to verify the integrity of the data. The default mode opens “transmit EAN-13 Check Digit”. Users can scan the code below to choose it.

| Transmit EAN-13 Check Digit(Default)* | Do Not Transmit EAN-13 Check Digit |
| --- | --- |

Add-On Code

An EAN-13 barcode can be augmented with a two-digit or five-digit add-on code to form a new one. In the examples below, the part surrounded by blue dotted line is an EAN-13 barcode while the part circled by red dotted line is add-on code.

| Enable 2-Digit Add-On Code | Disable 2-Digit Add-On Code(Default)* |
| --- | --- |
| Enable 5-Digit Add-On Code | Disable 5-Digit Add-On Code(Default)* |

EAN13 Addenda Separator

When this feature is enabled, there is a space between the barcode and addenda. When this feature is disabled, there is no space.

| Enable EAN13 Addenda Separator(Default)* | Disable EAN13 Addenda Separator |
| --- | --- |

Codabar

Restore Factory Defaults

Restore the Factory Defaults of Codabar

Enable/Disable Codabar

| Enable Codabar* | Disable Codabar |
| --- | --- |

Start/Stop Characters

There is a code like this <*Codebar*>, These hash keys refers to Start and Stop, you can set it whether the start and stop character transmit with the barcode.

| Transmit Codabar Character | Don’t Transmit Codabar Character(Default)* |
| --- | --- |

Code 39

Restore Factory Defaults

Restore the Factory Defaults of Code 39

Enable/Disable Code 39

| Enable Code 39(Default)* | Disable Code 39 |
| --- | --- |

Start/Stop Characters

There is a code like this <*Code 39*>, These hash keys refers to Start and Stop, you can set it whether the start and stop character transmit with the barcode.

| Transmit Coda 39 Character | Don’t Transmit Coda 39 Character(Default)* |
| --- | --- |

Enable/Disable Code 39 Full ASCII

The scanner can be configured to identify all ASCII characters by scanning the appropriate barcode below.

| Enable Code 39 Full ASCII | Disable Code 39 Full ASCII(Default)* |
| --- | --- |

Code 93

Restore Factory Defaults

Restore the Factory Defaults of Code 93

Enable/Disable Code 93

| Enable Code 93(Default)* | Disable Code 93 |
| --- | --- |

Code 128

Restore Factory Defaults

Restore the Factory Defaults of Code 128

Enable/Disable Code 128

| Enable Code 128(Default)* | Disable Code 128 |
| --- | --- |

GS1-128

Restore Factory Defaults

Restore the Factory Defaults of GS1-128

Enable/Disable GS1-128

| Enable GS1-128(Default)* | Disable GS1-128 |
| --- | --- |

UPC-A

Restore Factory Defaults

Restore the Factory Defaults of UPC-A

Enable/Disable UPC-A

| Enable UPC-A(Default)* | Disable UPC-A |
| --- | --- |

Transmit Check Digit

UPC-A is 12 digits in length with the last one as its check digit used to verify the integrity of the data. The default mode opens “transmit UPC-A Check Digit”. Users can scan the code below to choose it.

| Transmit UPC-A Check Digit(Default)* | Don’t transmit UPC-A Check Digit |
| --- | --- |

Add-on Code

An UPC-A barcode can be augmented with a two-digit or five-digit add-on code to form a new one. In the examples below, the part surrounded by blue dotted line is an UPC-A barcode while the part circled by red dotted line is add-on code.

| Enable 2-Digit Add-On Code | Disable 2-Digit Add-On Code(Default)* |
| --- | --- |
| Enable 5-Digit Add-On Code | Disable 5-Digit Add-On Code(Default)* |

UPC-A Addenda Separator

When this feature is enabled, there is a space between the barcode and addenda. When this feature is disabled, there is no space.

| Enable UPC-A Addenda Separator(Default)* | Disable UPC-A Addenda Separator |
| --- | --- |

UPC-E

Restore Factory Defaults

Restore the Factory Defaults of UPC-E

Enable/Disable UPC-E

| Enable UPC-E0(Default)* | Disable UPC-E0 |
| --- | --- |
| Enable UPC-E1 | Disable UPC-E1(Default)* |

Transmit Check Digit

UPC-E0 is 12 digits in length with the last one as its check digit used to verify the integrity of the data. The default mode opens “transmit UPC-E0 Check Digit”. Users can scan the code below to choose it.

| Transmit UPC-E0 Check Digit(Default)* | Don’t transmit UPC-E0 Check Digit |
| --- | --- |

Add-on Code

An UPC-E0 barcode can be augmented with a two-digit or five-digit add-on code to form a new one. In the examples below, the part surrounded by blue dotted line is an UPC-E0 barcode while the part circled by red dotted line is add-on code.

| Enable 2-Digit Add-On Code | Disable 2-Digit Add-On Code(Default)* |
| --- | --- |
| Enable 5-Digit Add-On Code | Disable 5-Digit Add-On Code(Default)* |

UPC-E0 Addenda Separator

When this feature is enabled, there is a space between the barcode and addenda. When this feature is disabled, there is no space.

| Enable UPC-E0 Addenda Separator(Default)* | Disable UPC-E0 Addenda Separator |
| --- | --- |

UPC-E0 Transfer to UPC-A

Users can set to transfer UPC-E to UPC-A according to your needs. The default closes this mode.

| Enable UPC-E0 Transfer to UPC-A | Disable UPC-E0 Transfer to UPC-A(Default)* |
| --- | --- |

Interleaved 2 of 5

Restore Factory Defaults

Restore the Factory Defaults of Interleaved 2 of 5

Enable/Disable Interleaved 2 of 5

| Enable Interleaved 2 of 5(Default)* | Disable Interleaved 2 of 5 |
| --- | --- |

Industrial 2 of 5

Restore Factory Defaults

Restore the Factory Defaults of Industrial 2 of 5

Enable/Disable Industrial 2 of 5

| Enable Industrial 2 of 5 | Disable Industrial 2 of 5(Default)* |
| --- | --- |

Matrix 2 of 5

Restore Factory Defaults

Restore the Factory Defaults of Matrix 2 of 5

Enable/Disable Matrix 2 of 5

| Enable Matrix 2 of 5 | Disable Matrix 2 of 5(Default)* |
| --- | --- |

MSI

Restore Factory Defaults

Restore the Factory Defaults of MSI

Enable/Disable MSI

| Enable MSI | Disable MSI(Default)* |
| --- | --- |

Telepen

Restore Factory Defaults

Restore the Factory Defaults of Telepen

Enable/Disable Telepen

| Enable Telepen | Disable Telepen(Default)* |
| --- | --- |

China Postal Code

Restore Factory Defaults

Restore the Factory Defaults of Telepen

Enable/Disable China Postal Code

| Enable Telepen | Disable Telepen(Default)* |
| --- | --- |

QR Code

Restore Factory Defaults

Restore the Factory Defaults of QR Code

Enable/Disable QR Code

| Enable QR Code(Default)* | Disable QR Code |
| --- | --- |

Data Matrix

Restore Factory Defaults

Restore the Factory Defaults of Data Matrix

Enable/Disable Data Matrix

| Enable Data Matrix(Default)* | Disable Data Matrix |
| --- | --- |

PDF 417

Restore Factory Defaults

Restore the Factory Defaults of PDF 417

Enable/Disable PDF417

| Enable PDF 417(Default)* | Disable PDF 417 |
| --- | --- |

Micro PDF 417

Restore Factory Defaults

Restore the Factory Defaults of Micro PDF 417

Micro PDF 417

| Enable Micro PDF 417(Default)* | Disable Micro PDF 417 |
| --- | --- |

Maxi code

Restore Factory Defaults

Restore the Factory Defaults of Maxicode

Enable/Disable Maxi Code

| Enable Maxicode | Disable Maxicode(Default)* |
| --- | --- |

Aztec

Restore Factory Defaults

Restore the Factory Defaults of Aztec

Enable/Disable Aztec

| Enable Aztec | Disable Aztec(Default)* |
| --- | --- |

Hanxin

Restore Factory Defaults

Hanxin(Default)

Enable/Disable Hanxin

| Enable Hanxin | Disable Hanxin(Default)* |
| --- | --- |

Chapter 7 Serial Commands

Introduction

The serial programming commands can be used in place of the programming barcodes. Both the serial programming commands and the programming barcodes will program the scanner. For complete descriptions and examples of each serial programming command, refer to the corresponding programming barcode in this manual.

The device must be set to a TTL-RS232 interface. The following commands can be sent via a PC COM port using terminal emulation software.

Command Format

The scanner can be applied to a command like”Prefix + Command”, users can choose characters or hexadecimal value as a command to control the scanner.

Prefix：[0X02][0X4D][0X0D]

Command：“Appendix C（Regular Command Instruction）”

Command Format：Prefix + Command

eg：the command of “Check the version”

Command: 091600.

Characters：[0X02][0XF0][0X03]091600.

Hexadecimal Value：02 F0 03 30 39 31 36 30 30 2E

Trigger a command

Send：02 F4 03

Don’t send：02 F5 03

An example of sending a command

To Click “Send as hex” to control the process of scanning, remember to confirm the default setting of serial port communication as below, and send the command in command area, then click “Send” command to start scanning.

Chapter 8 Appendix

Appendix A（Byte Code）

| 0 | 1 | 2 | 3 |
| --- | --- | --- | --- |

| 4 | 5 | 6 | 7 |
| --- | --- | --- | --- |

| 8 | 9 | A | B |
| --- | --- | --- | --- |

| C | D | E | F |
| --- | --- | --- | --- |

Appendix B（ASCII Table）

| HEX | Dec | Char |
| --- | --- | --- |
| 00 | 0 | NUL (Null char.) |
| 01 | 1 | SOH (Start of Header) |
| 02 | 2 | STX (Start of Text) |
| 03 | 3 | ETX (End of Text) |
| 04 | 4 | EOT (End of Transmission) |
| 05 | 5 | ENQ (Enquiry) |
| 06 | 6 | ACK (Acknowledgment) |
| 07 | 7 | BEL (Bell) |
| 08 | 8 | BS (Backspace) |
| 09 | 9 | HT (Horizontal Tab) |
| 0a | 10 | LF (Line Feed) |
| 0b | 11 | VT (Vertical Tab) |
| 0c | 12 | FF (Form Feed) |
| 0d | 13 | CR (Carriage Return) |
| 0e | 14 | SO (Shift Out) |
| 0f | 15 | SI (Shift In) |
| 10 | 16 | DLE (Data Link Escape) |
| 11 | 17 | DC1 (XON) (Device Control 1) |
| 12 | 18 | DC2 (Device Control 2) |
| 13 | 19 | DC3 (XOFF) (Device Control 3) |
| 14 | 20 | DC4 (Device Control 4) |
| 15 | 21 | NAK (Negative Acknowledgment) |
| 16 | 22 | SYN (Synchronous Idle) |
| 17 | 23 | ETB (End of Trans. Block) |
| 18 | 24 | CAN (Cancel) |
| 19 | 25 | EM (End of Medium) |
| 1a | 26 | SUB (Substitute) |

| HEX | Dec | Char |
| --- | --- | --- |
| 1b | 27 | ESC (Escape) |
| 1c | 28 | FS (File Separator) |
| 1d | 29 | GS (Group Separator) |
| 1e | 30 | RS (Request to Send) |
| 1f | 31 | US (Unit Separator) |
| 20 | 32 | SP (Space) |
| 21 | 33 | ! (Exclamation Mark) |
| 22 | 34 | " (Double Quote) |
| 23 | 35 | # (Number Sign) |
| 24 | 36 | $ (Dollar Sign) |
| 25 | 37 | % (Percent) |
| 26 | 38 | & (Ampersand) |
| 27 | 39 | ` (Single Quote) |
| 28 | 40 | ( (Right / Closing Parenthesis) |
| 29 | 41 | ) (Right / Closing Parenthesis) |
| 2a | 42 | * (Asterisk) |
| 2b | 43 | + (Plus) |
| 2c | 44 | , (Comma) |
| 2d | 45 | - (Minus / Dash) |
| 2e | 46 | . (Dot) |
| 2f | 47 | / (Forward Slash) |
| 30 | 48 | 0 |
| 31 | 49 | 1 |
| 32 | 50 | 2 |
| 33 | 51 | 3 |
| 34 | 52 | 4 |
| 35 | 53 | 5 |

| HEX | Dec | Char |
| --- | --- | --- |
| 36 | 54 | 6 |
| 37 | 55 | 7 |
| 38 | 56 | 8 |
| 39 | 57 | 9 |
| 3a | 58 | : (Colon) |
| 3b | 59 | ; (Semi-colon) |
| 3c | 60 | < (Less Than) |
| 3d | 61 | = (Equal Sign) |
| 3e | 62 | > (Greater Than) |
| 3f | 63 | ? (Question Mark) |
| 40 | 64 | @ (AT Symbol) |
| 41 | 65 | A |
| 42 | 66 | B |
| 43 | 67 | C |
| 44 | 68 | D |
| 45 | 69 | E |
| 46 | 70 | F |
| 47 | 71 | G |
| 48 | 72 | H |
| 49 | 73 | I |
| 4a | 74 | J |
| 4b | 75 | K |
| 4c | 76 | L |
| 4d | 77 | M |
| 4e | 78 | N |
| 4f | 79 | O |
| 50 | 80 | P |
| 51 | 81 | Q |

| HEX | Dec | Char |
| --- | --- | --- |
| 52 | 82 | R |
| 53 | 83 | S |
| 54 | 84 | T |
| 55 | 85 | U |
| 56 | 86 | V |
| 57 | 87 | W |
| 58 | 88 | X |
| 59 | 89 | Y |
| 5a | 90 | Z |
| 5b | 91 | [ (Left / Opening Bracket) |
| 5c | 92 | \ (Back Slash) |
| 5d | 93 | ] (Right / Closing Bracket) |
| 5e | 94 | ^ (Caret / Circum ﬂex) |
| 5f | 95 | _ (Underscore) |
| 60 | 96 | ' (Grave Accent) |
| 61 | 97 | a |
| 62 | 98 | b |
| 63 | 99 | c |
| 64 | 100 | d |
| 65 | 101 | e |
| 66 | 102 | f |
| 67 | 103 | g |
| 68 | 104 | h |
| 69 | 105 | i |
| 6a | 106 | j |
| 6b | 107 | k |
| 6c | 108 | l |
| 6d | 109 | m |

| HEX | Dec | Char |
| --- | --- | --- |
| 6e | 110 | n |
| 6f | 111 | o |
| 70 | 112 | p |
| 71 | 113 | q |
| 72 | 114 | r |
| 73 | 115 | s |
| 74 | 116 | t |
| 75 | 117 | u |
| 76 | 118 | v |
| 77 | 119 | w |
| 78 | 120 | x |
| 79 | 121 | y |
| 7a | 122 | z |
| 7b | 123 | { (Left/ Opening Brace) |
| 7c | 124 | ¦ (Vertical Bar) |
| 7d | 125 | } (Right/Closing Brace) |
| 7e | 126 | ~ (Tilde) |
| 7f | 127 | DEL (Delete) |

Appendix C（Regular Command Instructions）

| Function | Serial Port Instructions | Returned Value |
| --- | --- | --- |
| - Restore Factory Default | 0D0100. | 0D0100[0x06]. |
| - Check the Version | 0D1302?. | Software Version |
| - Scanning Command On | [0x02]T[0x03] | NA |
| - Scanning Command Off | [0x02]U[0x03] | NA |
| - Trigger Button Mode | 091A00. | 091A00[0x06]. |
| - Presentation Mode | 090901. | 090901[0x06]. |
| - Mobile Mode-Trigger Button | 091B00. | 091B00[0x06]. |
| - Mobile Mode-Presentation | 090902. | 090902[0x06]. |
| - Normal Image | 024B000. | 024B000[0x06]. |
| - Inverse Image | 024B001. | 024B001[0x06]. |
| - Normal+Inverse Image | 024B002. | 024B002[0x06]. |
| - Illumination on | 0501001. | 0501001[0x06]. |
| - Illumination off | 0501000. | 0501000[0x06]. |
| - Illumination Level 4 | 0401004. | 0401004[0x06]. |
| - Illumination Level 3 | 0401003. | 0401003[0x06]. |
| - Illumination Level 2 | 0401002. | 0401002[0x06]. |
| - Illumination Level 1 | 0401001. | 0401001[0x06]. |
| - Beeper On | 0502101. | 0502101[0x06]. |
| - Beeper Off | 0502100. | 0502100[0x06]. |
| - Low Beeper Tone | 05021D1. | 05021D1[0x06]. |
| - Middle Beeper Tone | 05021D2. | 05021D2[0x06]. |
| - High Beeper Tone | 05021D3. | 05021D3[0x06]. |
| - Beeper once | 05020E1. | 05020E1[0x06]. |
| - Beeper twice | 05020E2. | 05020E2[0x06]. |
| - Beeper three times | 05020E3. | 05020E3[0x06]. |
| - Beeper four times | 05020E4. | 05020E4[0x06]. |
| - Beeper five times | 05020E5. | 05020E5[0x06]. |
| - Beeper six times | 05020E6. | 05020E6[0x06]. |
| - No scanning delay | 080B080. | 080B080[0x06]. |
| - Scanning delay 500MS | 080B08500. | 080B08500[0x06]. |
| - Scanning delay 2000MS | 080B082000. | 080B082000[0x06]. |
| - USB HID-KBW | 091809. | NA |
| - Convert Case Off | 060D020. | 060D020[0x06]. |
| - All Upper Letter | 060D021. | 060D021[0x06]. |
| - All Lower Letter | 060D022. | 060D022[0x06]. |
| - USB COM Port Emulation | 060500130. | NA |
| - RS232 Interface | 091804. | NA |
| - Baud Rate 2400BPS | 0607023. | 0607023[0x06]. |
| - Baud Rate 4800BPS | 0607024. | 0607024[0x06]. |
| - Baud Rate 9600BPS | 0607025. | 0607025[0x06]. |
| - Baud Rate19200BPS | 0607026. | 0607026[0x06]. |
| - Baud Rate 38400BPS | 0607027. | 0607027[0x06]. |
| - Baud Rate 57600BPS | 0607028. | 0607028[0x06]. |
| - Baud Rate 115200BPS | 0607029. | 0607029[0x06]. |
| - N/8/1 | 0607032. | 0607032[0x06]. |
| - E/8/1 | 0607035. | 0607035[0x06]. |
| - O/8/1 | 0607038. | 0607038[0x06]. |
| - Add Return CR | 080500990D. | 080500990D[0x06]. |
| - Add Newline LF | 080500990A. | 080500990A[0x06]. |
| - Add Return&Newline CR+LF | 080500990D0A. | 080500990D0A[0x06]. |
| - Add TAB | 0805009909. | 0805009909[0x06]. |
| - Add prefix | 08040099XX. | 08040099XX[0x06]. |
| - Add Suffix | 08050099XX. | 08040099XX[0x06]. |
| - Save | 0D0400. | NA |
| - Not save | 0D0500. | NA |
| - Clear all prefix and suffix | 080404;080507. | NA |
| - Enable All Symbologies | 0201001. | 0201001[0x06]. |
| - Disable All Symbologies | 0201000. | 0201000[0x06]. |
| - EAN-8 default setting | 021400. | 021400[0x06]. |
| - Enable EAN-8 | 0214011. | 0214011[0x06]. |
| - Disable EAN-8 | 0214010. | 0214010[0x06]. |
| - Transmit EAN-8 Check Digit* | 0214021. | 0214021[0x06]. |
| - Do Not Transmit EAN-8 Check Digit | 0214020. | 0214020[0x06]. |
| - Enable EAN-8 2 Digit Add-On Code | 0214031. | 0214031[0x06]. |
| - Disable EAN-8 2 Digit Add-On Code | 0214030. | 0214030[0x06]. |
| - Enable EAN-8 5 Digit Add-On Code | 0214041. | 0214041[0x06]. |
| - Disable EAN-8 5 Digit Add-On Code | 0214040. | 0214040[0x06]. |
| - Enable EAN 8 Addenda Separator | 0214061. | 0214061[0x06]. |
| - Disable EAN 8 Addenda Separator | 0214060. | 0214060[0x06]. |
| - EAN-13 Default Setting | 021300. | 021300[0x06]. |
| - Enable EAN-13 | 0213011. | 0213011[0x06]. |
| - Disable EAN-13 | 0213010. | 0213010[0x06]. |
| - Transmit EAN-13 Check Digit* | 0213021. | 0213021[0x06]. |
| - Do Not Transmit EAN-13 Check Digit | 0213020. | 0213020[0x06]. |
| - Enable EAN-13 2 Digit Add-On Code | 0213031. | 0213031[0x06]. |
| - Disable EAN-13 2 Digit Add-On Code | 0213030. | 0213030[0x06]. |
| - Enable EAN-13 5 Digit Add-On Code | 0213041. | 0213041[0x06]. |
| - Disable EAN-13 5 Digit Add-On Code | 0213040. | 0213040[0x06]. |
| - Enable EAN13 Addenda Separator | 0213061. | 0213061[0x06]. |
| - Disable EAN13 Addenda Separator | 0213060. | 0213060[0x06]. |
| - Codabar Default Setting | 020200. | 020200[0x06]. |
| - Enable Codabar | 0202011. | 0202011[0x06]. |
| - Disable Codabar | 0202010. | 0202010[0x06]. |
| - Enable Codabar Start/Stop Characters | 0202021. | 0202021[0x06]. |
| - Disable Codabar start/stop characters | 0202020. | 0202020[0x06]. |
| - Code 39 Default Setting | 020300. | 020300. |
| - Enable Code 39 | 0203011. | 0203011[0x06]. |
| - Disable Code 39 | 0203010. | 0203010[0x06]. |
| - Enable Coda 39 Start/Stop Characters | 0203051. | 0203051[0x06]. |
| - Disable Coda 39 start/stop characters | 0203050. | 0203050[0x06]. |
| - Enable Code 39 Full ASCII | 0203021. | 0203021[0x06]. |
| - Disable Code 39 Full ASCII | 0203020. | 0203020[0x06]. |
| - Code 93 Default Setting | 020D00. | 020D00[0x06]. |
| - Enable Code 93 | 020D011. | 020D011[0x06]. |
| - Disable Code 93 | 020D010. | 020D010[0x06]. |
| - Code128 Default Setting | 020A00. | 020A00[0x06]. |
| - Enable Code 128 | 020A011. | 020A011[0x06]. |
| - Disable Code128 | 020A010. | 020A010[0x06]. |
| - GS1-128 Default Setting | 020B01. | 020B01[0x06]. |
| - Enable GS1-128 | 020B001. | 020B001[0x06]. |
| - Disable GS1-128 | 020B000. | 020B000[0x06]. |
| - UPC-A Default Setting | 021100. | 021100[0x06]. |
| - Enable UPC-A | 0211011. | 0211011[0x06]. |
| - Disable UPC-A | 0211010. | 0211010[0x06]. |
| - Transmit UPC-A Check Digit* | 0211021. | 0211021[0x06]. |
| - Do Not Transmit UPC-A Check Digit | 0211020. | 0211020[0x06]. |
| - Enable UPC-A 2 Digit Add-On Code | 0211041. | 0211041[0x06]. |
| - Disable UPC-A 2 Digit Add-On Code | 0211040. | 0211040[0x06]. |
| - Enable UPC-A 5 Digit Add-On Code | 0211051. | 0211051[0x06]. |
| - Enable UPC-A 5 Digit Add-On Code | 0211050. | 0211050[0x06]. |
| - Enable UPC-A Addenda Separator | 0211071. | 0211071[0x06]. |
| - Disable UPC-A Addenda Separator | 0211070. | 0211070[0x06]. |
| - UPC-E Default Setting | 021200. | 021200[0x06]. |
| - Enable UPC-E0 | 0212011. | 0212011[0x06]. |
| - Disable UPC-E0 | 0212010. | 0212010[0x06]. |
| - Enable UPC-E1 | 0212021. | 0212021[0x06]. |
| - Disable UPC-E1 | 0212020. | 0212020[0x06]. |
| - Enable UPC-E0 Transfer to UPC-A | 0212031. | 0212031[0x06]. |
| - Disable UPC-E0 Transfer to UPC-A | 0212030. | 0212030[0x06]. |
| - Transmit UPC-E0 Check Digit* | 0212041. | 0212041[0x06]. |
| - Do Not Transmit UPC-E0 Check Digit | 0212040. | 0212040[0x06]. |
| - Enable UPC-E0 2 Digit Add-On Code | 0212061. | 0212061[0x06]. |
| - Disable UPC-E0 2 Digit Add-On Code | 0212060. | 0212060[0x06]. |
| - Enable UPC-E0 5 Digit Add-On Code | 0212071. | 0212071[0x06]. |
| - Disable UPC-E0 5 Digit Add-On Code | 0212070. | 0212070[0x06]. |
| - Enable UPC-E0 Addenda Separator | 0212091. | 0212091[0x06]. |
| - Disable UPC-E0 Addenda Separator | 0212090. | 0212090[0x06]. |
| - Interleaved 2 of 5 Default Setting | 020400. | 020400[0x06]. |
| - Enable Interleaved 2 of 5 | 0204011. | 0204011[0x06]. |
| - Disable Interleaved 2 of 5 | 0204010. | 0204010. |
| - Industrial 2 of 5 Default Setting | 020600. | 020600[0x06]. |
| - Enable Industrial 2 of 5 | 0206011. | 0206011[0x06]. |
| - Disable Industrial 2 of 5 | 0206010. | 0206010[0x06]. |
| - Matrix 2 of 5 Default Setting | 020800. | 020800[0x06]. |
| - Enable Matrix 2 of 5 | 0208011. | 0208011[0x06]. |
| - Disable Matrix 2 of 5 | 0208010. | 0208010[0x06]. |
| - MSI Default Setting | 020E00. | 020E00[0x06]. |
| - Enable MSI | 020E011. | 020E011[0x06]. |
| - Disable MSI | 020E010. | 020E010[0x06]. |
| - Telepen Default Setting | 021000. | 021000[0x06]. |
| - Enable Telepen | 0210011. | 0210011[0x06]. |
| - Disable Telepen | 0210010. | 0210010[0x06]. |
| - China Postal Default Setting | 021800. | 021800[0x06]. |
| - Enable China Postal | 0218011. | 0218011[0x06]. |
| - Disable China Postal | 0218010. | 0218010[0x06]. |
| - QR Code Default Setting | 023700. | 023700[0x06]. |
| - Enable QR Code | 0237011. | 0237011[0x06]. |
| - Disable QR Code | 0237010. | 0237010[0x06]. |
| - Data Matrix Default Setting | 023600. | 023600[0x06]. |
| - Enable Data Matrix | 0236011. | 0236011[0x06]. |
| - Disable Data Matrix | 0236010. | 0236010[0x06]. |
| - PDF 417 Default Setting | 021F00. | 021F00[0x06]. |
| - Enable PDF 417 | 021F011. | 021F011[0x06]. |
| - Disable PDF 417 | 021F010. | 021F010[0x06]. |
| - Micro PDF 417 Default Setting | 022000. | 022000[0x06]. |
| - Enable Micro PDF 417 | 0220011. | 0220011[0x06]. |
| - Disable Micro PDF 417 | 0220010. | 0220010[0x06]. |
| - Maxicode Default Setting | 023400. | 023400[0x06]. |
| - Enable Maxicode | 0234011. | 0234011[0x06]. |
| - Disable Maxicode | 0234010. | 0234010[0x06]. |
| - Aztec Default Setting | 023300. | 023300[0x06]. |
| - Enable Aztec | 0233011. | 0233011[0x06]. |
| - Disable Aztec | 0233010. | 0233010[0x06]. |
| - Hanxin Default Setting | 023800. | 023800[0x06]. |
| - Enable Hanxin | 0238011. | 0238011[0x06]. |
| - Disable Hanxin | 0238010. | 0238010[0x06]. |